LB611 	LB611

2025 	2025

LEGISLATURE OF NEBRASKA

ONE HUNDRED NINTH LEGISLATURE

FIRST SESSION

LEGISLATIVE BILL 611

Introduced by Urban Affairs Committee: McKinney, 11, Chairperson;

Cavanaugh, J., 9; Clouse, 37; Quick, 35; Rountree, 3;

Sorrentino, 39.

Read first time January 22, 2025

Committee: Urban Affairs

1 	A BILL FOR AN ACT relating to buildings; to amend sections 81-1608,

2 	81-1611, and 81-1614, Reissue Revised Statutes of Nebraska, and

3 	sections 71-6403, 71-6406, 72-804, 72-805, and 72-806, Revised

4 	Statutes Cumulative Supplement, 2024; to adopt updates to building

5 	and energy codes; and to repeal the original sections.

6 	Be it enacted by the people of the State of Nebraska,

-1-

LB611 	LB611

2025 	2025

1 	Section 1. Section 71-6403, Revised Statutes Cumulative Supplement,

2 	2024, is amended to read:

3 	71-6403 (1) There is hereby created the state building code. The

4 	Legislature hereby adopts by reference:

5 	(a) The International Building Code (IBC), chapter 13 of the 2021

6 	edition, and all but such chapter of the 2018 edition, except section

7 	101.4.3 and chapter 29, published by the International Code Council,

8 	except that (i) section 305.2.3 applies to a facility having twelve or

9 	fewer children and (ii) section 310.4.1 applies to a care facility for

10 	twelve or fewer persons;

11 	(b) The International Residential Code (IRC), chapter 11 of the 2021

12 	edition, and all but such chapter of the 2018 edition, except section

13 	R313 and chapters 25 through 33, published by the International Code

14 	Council;

15 	(c) The International Existing Building Code, 2018 edition, except

16 	section 809, published by the International Code Council; and

17 	(d) The Uniform Plumbing Code, 2018 edition, designated by the

18 	American National Standards Institute as an American National Standard.

19 	(2) The codes adopted by reference in subsection (1) of this section

20 	and the minimum standards for radon resistant new construction adopted

21 	under section 76-3504 shall constitute the state building code except as

22 	amended pursuant to the Building Construction Act or as otherwise

23 	authorized by state law.

24 	Sec. 2. Section 71-6406, Revised Statutes Cumulative Supplement,

25 	2024, is amended to read:

26 	71-6406 (1)(a) Any county, city, or village may enact, administer,

27 	or enforce a local building or construction code if or as long as such

28 	county, city, or village:

29 	(i) Adopts the state building code; or

30 	(ii) Adopts a building or construction code that conforms generally

31 	with the state building code.

-2-

LB611 	LB611

2025 	2025

1 	(b) If a county, city, or village does not adopt a code as

2 	authorized under subdivision (a) of this subsection within two years

3 	after an update to the state building code, the state building code shall

4 	apply in the county, city, or village, except that such code shall not

5 	apply to construction on a farm or for farm purposes.

6 	(2) A local building or construction code shall be deemed to conform

7 	generally with the state building code if it:

8 	(a) Adopts a special or differing building standard by amending,

9 	modifying, or deleting any portion of the state building code in order to

10 	reduce unnecessary costs of construction, increase safety, durability, or

11 	efficiency, establish best building or construction practices within the

12 	county, city, or village, or address special local conditions within the

13 	county, city, or village;

14 	(b) Adopts any supplement, new edition, appendix, or component or

15 	combination of components of the state building code;

16 	(c) Adopts any of the following:

17 	(i) Chapter 13 of the 2021 edition of the International Building

18 	Code;

19 	(ii) (i) Section 305 or 310 of the 2018 edition of the International

20 	Building Code without the exceptions described in subdivision (1)(a) of

21 	section 71-6403;

22 	(iii) (ii) Section 101.4.3 or any portion of chapter 29 of the 2018

23 	edition of the International Building Code;

24 	(iv) Chapter 11 of the 2021 edition of the International Residential

25 	Code;

26 	(v) (iii) Section R313 or any portion of chapters 25 through 33 of

27 	the 2018 edition of the International Residential Code; or

28 	(vi) (iv) Section 809 of the 2018 edition of the International

29 	Existing Building Code;

30 	(d) Adopts a plumbing code, an electrical code, a fire prevention

31 	code, or any other standard code as authorized under section 14-419,

-3-

LB611 	LB611

2025 	2025

1 	15-905, 18-132, or 23-172;

2 	(e) Adopts a local energy code as authorized under section 81-1618;

3 	or

4 	(f) Adopts minimum standards for radon resistant new construction

5 	which meet the minimum standards adopted under section 76-3504.

6 	(3) A local building or construction code shall not be deemed to

7 	conform generally with the state building code if it:

8 	(a) Includes a prior edition of any component or combination of

9 	components of the state building code; or

10 	(b) Does not include minimum standards for radon resistant new

11 	construction that meet the minimum standards adopted under section

12 	76-3504.

13 	(4) A county, city, or village shall notify the Department of

14 	Environment and Energy if it amends or modifies its local building or

15 	construction code in such a way as to delete any portion of (a) chapter

16 	13 of the 2021 2018 edition of the International Building Code or (b)

17 	chapter 11 of the 2021 2018 edition of the International Residential

18 	Code. The notification shall be made within thirty days after the

19 	adoption of such amendment or modification.

20 	(5) A county, city, or village shall not adopt or enforce a local

21 	building or construction code other than as provided by this section.

22 	(6) A county, city, or village which adopts or enforces a local

23 	building or construction code under this section shall regularly update

24 	its code. For purposes of this section, a code shall be deemed to be

25 	regularly updated if the most recently enacted state building code or a

26 	code that conforms generally with the state building code is adopted by

27 	the county, city, or village within two years after an update to the

28 	state building code.

29 	(7) A county, city, or village may adopt amendments for the proper

30 	administration and enforcement of its local building or construction code

31 	including organization of enforcement, qualifications of staff members,

-4-

LB611 	LB611

2025 	2025

1 	examination of plans, inspections, appeals, permits, and fees. Any

2 	amendment adopted pursuant to this section shall be published separately

3 	from the local building or construction code. Any local building or

4 	construction code adopted under subdivision (1)(a) of this section or the

5 	state building code if applicable under subdivision (1)(b) of this

6 	section shall be the legally applicable code regardless of whether the

7 	county, city, or village has provided for the administration or

8 	enforcement of its local building or construction code under this

9 	subsection.

10 	(8) A county, city, or village which adopts one or more standard

11 	codes as part of its local building or construction code under this

12 	section shall keep at least one copy of each adopted code, or portion

13 	thereof, for use and examination by the public in the office of the clerk

14 	of the county, city, or village prior to the adoption of the code and as

15 	long as such code is in effect.

16 	(9) Notwithstanding the provisions of the Building Construction Act,

17 	a public building of any political subdivision shall be built in

18 	accordance with the applicable local building or construction code. Fees,

19 	if any, for services which monitor a builder's application of codes shall

20 	be negotiable between the political subdivisions involved, but such fees

21 	shall not exceed the actual expenses incurred by the county, city, or

22 	village doing the monitoring.

23 	Sec. 3. Section 72-804, Revised Statutes Cumulative Supplement,

24 	2024, is amended to read:

25 	72-804 (1) Any new state building shall meet or exceed the

26 	requirements of the 2021 2018 International Energy Conservation Code

27 	published by the International Code Council.

28 	(2) Any new lighting, heating, cooling, ventilating, or water

29 	heating equipment or controls in a state-owned building and any new

30 	building envelope components installed in a state-owned building shall

31 	meet or exceed the requirements of the 2021 2018 International Energy

-5-

LB611 	LB611

2025 	2025

1 	Conservation Code.

2 	(3) 	The 	State 	Building 	Administrator 	of 	the 	Department 	of

3 	Administrative Services, 	in 	consultation 	with 	the 	Department 	of

4 	Environment and Energy, may specify:

5 	(a) A more recent edition of the International Energy Conservation

6 	Code;

7 	(b) Additional energy efficiency or renewable energy requirements

8 	for buildings; and

9 	(c) Waivers of specific requirements which are demonstrated through

10 	life-cycle cost analysis to not be in the state's best interest. The

11 	agency receiving the funding shall be required to provide a life-cycle

12 	cost analysis to the State Building Administrator.

13 	Sec. 4. Section 72-805, Revised Statutes Cumulative Supplement,

14 	2024, is amended to read:

15 	72-805 The 2021 2018 International Energy Conservation Code,

16 	published by the International Code Council, applies to all new buildings

17 	constructed in whole or in part with state funds after the effective date

18 	of this act after July 1, 2020. The Department of Environment and Energy

19 	shall review building plans and specifications necessary to determine

20 	whether a building will meet the requirements of this section. The

21 	department shall provide a copy of its review to the agency receiving

22 	funding. The agency receiving the funding shall verify that the building

23 	as constructed meets or exceeds the code. The verification shall be

24 	provided to the department. The Director of Environment and Energy may,

25 	in consultation with the State Building Administrator of the Department

26 	of Administrative Services, adopt and promulgate rules and regulations to

27 	carry out this section.

28 	Sec. 5. Section 72-806, Revised Statutes Cumulative Supplement,

29 	2024, is amended to read:

30 	72-806 The enforcement provisions of Chapter 1 of the 2021 2018

31 	International Energy Conservation Code, published by the International

-6-

LB611 	LB611

2025 	2025

1 	Code Council, shall not apply to buildings subject to section 72-804.

2 	Sec. 6. Section 81-1608, Reissue Revised Statutes of Nebraska, is

3 	amended to read:

4 	81-1608 The Legislature finds that consumers have an expectation

5 	that newly built houses or buildings they buy meet uniform energy

6 	efficiency standards. Therefor, the Legislature finds that there is a

7 	need to adopt the 2021 2018 International Energy Conservation Code,

8 	published by the International Code Council, in order (1) to ensure that

9 	a minimum energy efficiency standard is maintained throughout the state,

10 	(2) to harmonize and clarify energy building code statutory references,

11 	(3) to ensure compliance with the federal Energy Policy Act of 1992, (4)

12 	to increase energy savings for all Nebraska consumers, especially low-

13 	income Nebraskans, (5) to reduce the cost of state programs that provide

14 	assistance to low-income Nebraskans, (6) to reduce the amount of money

15 	expended to import energy, (7) to reduce the growth of energy

16 	consumption, (8) to lessen the need for new power plants, and (9) to

17 	provide training for local code officials and residential and commercial

18 	builders who implement the 2021 2018 International Energy Conservation

19 	Code.

20 	Sec. 7. Section 81-1611, Reissue Revised Statutes of Nebraska, is

21 	amended to read:

22 	81-1611 The Legislature hereby adopts the 2021 2018 International

23 	Energy Conservation Code published by the International Code Council as

24 	the Nebraska Energy Code. The Director of Environment and Energy may

25 	adopt regulations specifying alternative standards for building systems,

26 	techniques, equipment designs, or building materials that shall be deemed

27 	equivalent 	to 	the 	Nebraska Energy 	Code. 	Regulations 	specifying

28 	alternative standards may be deemed equivalent to the Nebraska Energy

29 	Code and may be approved for general or limited use if the use of such

30 	alternative standards would not result in energy consumption greater than

31 	would result from the strict application of the Nebraska Energy Code.

-7-

LB611 	LB611

2025 	2025

1 	Sec. 8. Section 81-1614, Reissue Revised Statutes of Nebraska, is

2 	amended to read:

3 	81-1614 The Nebraska Energy Code shall apply to all new buildings,

4 	or renovations of or additions to any existing buildings, on which

5 	construction is initiated on or after the effective date of this act July

6 	1, 2020.

7 	Sec. 9.  Original sections 81-1608, 81-1611, and 81-1614, Reissue

8 	Revised Statutes of Nebraska, and sections 71-6403, 71-6406, 72-804,

9 	72-805, and 72-806, Revised Statutes Cumulative Supplement, 2024, are

10 	repealed.

-8-